mINT13.SYS is an MS-DOS device driver which implements a memory cache

for data referenced on IBM-PC XT/AT Hard disks. It caches at the

"INT 13H" level and caches tracks.

NOTE WARNING: There is a "re-boot" bug in RAMDrive version 1.16. When

INT13 and RAMDrive are both installed, it is possible

that Ctrl-Alt-Del will not work. This problem is "fixed"

by updating your RAMDrive to version 1.17 or later.

IN CONFIG.SYS:

device = [d:][path]int13.sys [bbbb] [/e | /a] [/d] [/wt:on] [/wc:off]

[/t:nnnnn] [/r:on] [/c:on]

bbbb  First numeric argument, if present, is cache size

in K bytes. Default value is 256. Min is 128. Max

is 4096 (4 Meg).

/e    Specifies that PC AT Extended Memory is to be used.

It is an error if /E is specified on a machine other

than an IBM PC AT. /E is the default.

NOTE: There is 1k of INT13 overhead. That is to say,

if there are 512k bytes of extended memory, there

will be 511k bytes available for assignment to INT13.

/a    Specifies that Above Board memory is to be used. It

is an error if the above board device driver is not

present.

Neither /A or /E Specifies /E

NOTE: THESE ARE THE ONLY CONFIGURATIONS. You must either have an

Above Board (or compatible), or you must have extended memory

on an IBM PC-AT (or compatible).

/d    Disable caching. Causes INT13 to come up with caching

disabled (default is enabled).

/wt:on Enable write through. When INT13 is

caching write information, it is a good idea to imply

a flush of the cache on some operations so that in

case of a crash or power failure the information in

the cache which is not on the disk will not be lost.

/wt:on enables write through on full track INT 13s which

are to tracks not currently in the cache. /wt:off

disables it. INT13 is faster with write through

disabled, at the expense of there being a bigger risk of

loosing data. /wt:on IS NOT a substitute for flushing

before a re-boot!!!! The write through mechanism is far

from perfect, all it is is a risk REDUCER, not a risk

eliminator. Write through is off by default.

/wc:off - Disable write caching. There is risk when

caching "dirty" information that the system will crash,

or be re-booted, or be turned off before this

information can be written to the disk. This may

corrupt the disk. This risk can be ELIMINATED, at the

expense of cache performance, by NOT caching any dirty

information. /wc:off disables the caching of dirty

information, eliminating the risk. Write caching is

on by default.

/t:nnnnn - Set the auto flush interval. INT13 listens on the

system timer to note the passage of time and "age" the

dirty information in the cache. Every nnnnn ticks,

the cache is flushed. The timer ticks 18.2 times a

second.

nnnnn	 |

===========================================

18	 |     Flush every second

1092	 |     Flush every minute

5460	 |     Flush every 5 minutes

10920	 |     Flush every 10 minutes

21840	 |     Flush every 20 minutes

32760	 |     Flush every 30 minutes

65520	 |     Flush every hour

The default setting of nnnnn is 1092 or every minute.

nnnnn = 0 causes a wait for 65536 ticks which is a

little over an hour. The max value for nnnnn is 65535.

/r:on - Enable re-boot flush.

This enables the flush on re-boot logic. The default

is /r:off. This enables a flush in the Ctrl-Alt-Del

keyboard re-boot logic. NOTE WARNING DANGER!!!!!!!!

Enabling this feature can cause strange system behavior.

You will have to type Ctrl-Alt-Del twice at least. This

can adversly effect other software in the system.

The result of this can be very unpredictable.

/c:on - Enable all cache on reads.

Normally INT13 does not cache EVERY I/O. Whenever

it sees a full track I/O which is not currently in

the cache, it DOES NOT cache that track. This is

an optimization for "typical" operation, and actually

increases performance. This is the default setting

(/c:off). There may be some cases where it is desirable

that ALL reads be cached. One example is that you are

"loading" the cache prior to locking it with FLUSH13 /l.

With /c:off, some pieces of what you're trying to load

may not get into the cache. Another example is that

you continually access in a sequential manner (like

program load) some large file which happens to be

contiguous on the disk. Again, there may be some "piece"

of the file which does not get into the cache with

/c:off. /c:on enables the caching of ALL reads.

NOTE: The same "don't bother caching operations which

are full track and not in the cache" applies

to writes as well. /c has NO EFFECT on this

behavior however. /c only effects read operations.

MESSAGES:

INT13: Above Board Memory Manager not present

The /A switch was given but INT13 could not detect the presence of the

Above Board memory manager. You need to have a

device = EMM.SYS

line in your CONFIG.SYS file before you have any device = int13.sys

lines. INT13 will install a driver, but it will be non-functional.

INT13: Above Board Memory Status shows error

During the process of trying to set up the cache in Above Board memory

an error was detected. Run the Above Board Confidence test to test

your Above Board memory. INT13 will install a driver, but it will be

non-functional.

INT13: Computer must be PC-AT, or PC-AT compatible

The /E switch can only be given on an IBM PC AT or an IBM PC AT

compatible computer that has the FCH model byte (byte at FFFF:000E).

INT13 will install a driver, but it will be non-functional.

INT13: No extended memory available

Your system has NO memory for RAMDrive drives.

INT13 will install a driver, but it will be non-functional.

INT13: Insufficient memory

Your system has some memory available for INT13 cache, but not enough

to set up a driver. INT13 will install a driver, but it will be non-functional.

INT13: Invalid parameter

You specified too many parameters, your numeric parameter is not

valid, you specified conflicting or too many switches. Edit your CONFIG.SYS

file and fix the INT13 line. INT13 will install a driver, but it will

be non-functional.

INT13: Incorrect DOS version

INT13 only runs on 2.X and 3.X versions of DOS.

INT13 will install a driver, but it will be non-functional.

INT13: I/O error accessing cache memory

During the set up of the INT13 cache, an error was detected trying to

access the cache memory. Run any memory tests you have that will

exercise your extended or expanded memory.

INT13 will install a driver, but it will be non-functional.

INT13: No hardfiles on system

INT13 could not find any hardfiles on your system. Only hardfiles

which are compatible with IBM hardfiles at the ROM BIOS INT 13H

level can be supported. INT13 will install a driver, but it will

be non-functional.

INT13: Too many bytes per track on hardfile

One of the hardfiles on your system defined a very large track.

This track is too large for INT13 to be able to cache it.

INT13 will install a driver, but it will be non-functional.

Microsoft INT13 Cache version Y.YY

INT13 Header message, Y.YY is the version of INT13.

Cache size: nnnnk in UUUUUU Memory

Room for tttt tracks of ssss sectors each

This is an informational message from INT13 telling you how many Kilo Bytes

of memory were assigned to the cache and what type of memory it is, how many

tracks this allows to be buffered and how many sectors there are per track.

NOTE: There is nothing to "prevent" you from having two device = INT13.SYS

lines in your CONFIG.SYS file, but you should not do this. Very

unpredictable behavior will occur, and FLUSH13 will only "talk" to

one of them.

INT13 has several behavior aspects that can be changed. This is done

with the FLUSH13 utility.

FLUSH13 [/s|/sx|/sr] [/d|/e] [/l|/u] [/i] [/f] [/wt:on|/wt:off]

[/wc:on|/wc:off] [/t:nnnnn] [/r:on|/r:off] [/c:on|/c:off]

No arguments - This causes FLUSH13 to flush out any "dirty"

tracks in the INT13 cache. A "dirty" track is one

which has been written into the cache, but not yet

written to the disk. This invokation causes all dirty tracks

to be written out to the disk so that the system can

be re-booted or turned off. NOTE: FAILURE TO FLUSH

THE CACHE BEFORE A RE-BOOT OR POWER OFF CAN CAUSE THE

INFORMATION ON THE HARDFILE TO BE CORRUPTED.

/f	- Flush. Same as the no arguments case, but allows you to

perform the flush and do something else (like /s).

/i	- Flush and invalidate. This is the same as the no argument

case except that all of the information in the cache

is also discarded. This makes the cache EMPTY.

/d	- Disable caching. This causes all dirty cache information

to be flushed and all caching to stop.

/e	- Enable caching. This causes caching to be enabled after

a previous /d disable. When INT13 is started it is enabled.

/l	- Lock the cache. This causes all dirty information to be

flushed, and the cache contents to be locked in the cache.

When in this mode the locked elements will not be discarded

to make room for new tracks. This can be used

to "load" the cache with desired things. For instance if

you use the "foobar" program a lot, you can run foobar,

causing it to be loaded into the cache, then lock the cache.

This causes the foobar program to always be in the cache.

You may lock the cache as many times as you want. Each lock

causes the current information (including any previously

locked information) to be locked.

NOTE: Information in a locked cache is READ ONLY!! Any write

operation on information in a locked cache causes the

information to be unlocked.

/u	- Unlock the cache. This undoes a previous /l and returns

the cache to normal operation.

/s	- Print status. This displays the settings of the setable

device parameters.

/sx	- Print extended status. Same as /s, only additional

Statistical information is also given.

/sr	- Reset statistics. Same as /sx, only the additional

Statistical information is reset to 0.

/wt:on off - Enable or Disable write through. When INT13 is caching

write information, it is a good idea to imply a flush of

the cache on some operations so that in case of a crash or

power failure the information in the cache which is not on

the disk will not be lost. /wt:on enables write through on full

track INT 13s which are to tracks not currently in the cache.

/wt:off disables it. INT13 is faster with write through

off, at the expense of there being a bigger risk of

loosing data. /wt:on IS NOT a substitute for flushing before

a re-boot!!!! This write through mechanism is far from perfect,

all it is is a risk REDUCER, not a risk eliminator. /wt:off

is the setting when INT13 is started.

/wc:on off - Enable or Disable write caching. There is risk when

caching "dirty" information that the system will crash,

or be re-booted, or be turned off before this information

can be written to the disk. This may corrupt the disk.

This risk can be ELIMINATED, at the expense of cache

performance, by NOT caching any dirty information.

/wc:off disables the caching of dirty information,

eliminating the risk. /wc:on enables the caching of dirty

information. /wc:on is the default when INT13 is started.

WARNING: You must be careful to flush the cache before

re-booting the system, or turning it off if /wc:on is selected.

You should also be careful to disable the cache (/d), or do

/wc:off before running any program under development which

has a chance of crashing due to bugs.

NOTE: When /wc:off is selected, write info CAN get into

the cache (when the write is to a track which is currently

in the cache). The difference is that this "dirty" information

is IMMEDIATELY written out to the disk instead of being

held in the cache in the "dirty" state. When the write is

to a track that is not in the cache, it will be passed

through to the disk without being cached.

/t:nnnnn - Set the auto flush interval. INT13 listens on the system

timer to note the passage of time and "age" the dirty

information in the cache. Every nnnnn ticks, the cache is

flushed. The timer ticks 18.2 times a second.

nnnnn	 |

===========================================

18	 |     Flush every second

1092	 |     Flush every minute

5460	 |     Flush every 5 minutes

10920	 |     Flush every 10 minutes

21840	 |     Flush every 20 minutes

32760	 |     Flush every 30 minutes

65520	 |     Flush every hour

The default setting of nnnnn is 1092 or every minute.

NOTE: There is no way to "disable" this tick aging. Setting

nnnnn = 0 causes a wait for 65536 ticks which is a

little over an hour. The max value for nnnnn is 65535.

Disabling the cache (/d), or turning write caching

off (/wc:off) effectively prevents the aging from

doing anything as there is never anything to flush

in these cases. Setting very low values of nnnnn

should be avoided as it places a lot of overhead into

the timer interrupt service. Rather than set low values,

it is better to just turn off write caching (/wc:off).

NOTE: As stated above, the max value for nnnnn is 65535. It

should be noted however that FLUSH13 DOES NOT object if

you specify a number larger than this! It will simply

use only the low 16 bits of the number.

/r:on off - En/Disable reboot flush.

INT13 has a provision for detecting Ctrl-Alt-Del user

reboots. /r:on enables a flush of the cache at this time

to prevent the disks from being corrupted. The default

setting is /r:off. NOTE WARNING DANGER!!!!! Enabling

this feature can prevent disks from being damaged BUT

the mechanism has flaws. For one, you will have to hit

Ctrl-Alt-Del a second time to get the system to reboot.

YOU MUST NOT POUND ON THE KEY. You will crash the system if

you do. Hit the key ONCE, if the system re-boots, fine. If

there is info to flush out of the cache, the drive light

will come on and the system will probably NOT reboot. WAIT

until the drive light is OFF before hitting Ctrl-Alt-Del

again. This feature of INT13 MAY NOT WORK with other

software in the system. USER BEWARE!!!!!!!!!!!!!!!!!!!

/c:on off - En/Disable all cache on reads.

Normally INT13 does not cache EVERY I/O. Whenever

it sees a full track I/O which is not currently in

the cache, it DOES NOT cache that track. This is

an optimization for "typical" operation, and actually

increases performance. This is the default setting

(/c:off). There may be some cases where it is desirable

that ALL reads be cached. One example is that you are

"loading" the cache prior to locking it with FLUSH13 /l.

With /c:off, some pieces of what you're trying to load

may not get into the cache. Another example is that

you continually access in a sequential manner (like

program load) some large file which happens to be

contiguous on the disk. Again, there may be some "piece"

of the file which does not get into the cache with

/c:off. /c:on enables the caching of ALL reads.

NOTE: The same "don't bother caching operations which

are full track and not in the cache" applies

to writes as well. /c has NO EFFECT on this

behavior however. /c only effects read operations.

If invalid or conflicting arguments are given, FLUSH13 prints a usage line.